# IndigoVision

How to add Vemotion Cameras to IndigoVision Control Centre:

Prerequisites:

VProxy: min version 2.2.2.4

Tested with Control Centre Ultra – Version 18.1.1

Methods:

There are three different methods for supporting IndigoVision:

- Is to simply use the advanced configuration features withing Control Centre Ultra

- is to listen on multiple ports in the VProxy and rely on Web Service Discovery for automatic detection for each port instance.

- Is to listen on multiple IP addresses but a single port, and manually enter each camera as a different address. (This is only supported on the Linux version the of VProxy.)

Method A – Trust IndigoVision Config tools

- You will need a user ready prepared with access to all the necessary Vemotion Streams.

- Restart indigoVision Control Centre.

This is required so that any discovered devices are cleared from the cache (which sometimes prevents new devices being discovered).

- Select “Setup – Top Site – Add Devices…”

(You can first check in “Visible Devices” if the Vemotion VProxy is visible but don’t worry if it isn’t.)

- Check “Add as cloneable” *important set.

Now select the Vemotion device or its Name and IP address manually

- Access the device properties

…and change the “Device Access” to use the correct Username Password as configured in the Vemotion Streaming Server

You must OK, this change before doing anything else.

- Now re-open the “Properties…” panel and go to “Live Video”

Uncheck “Use same settings as parent site” *important, and then select the desired Vemotion Stream by “ONVIF Profile Token”.

Note: if you’ve configured the Vemotion Streaming Server to supply single different streams for multiple users then you can skip this set.

- You now have a working Vemotion Stream!

- To add another Vemotion Stream, simply “clone” the first stream.

- And then edit the properties of the new stream.

- Change the “Live Video” – “Default stream setting” to a different Stream.

Don’t for get to uncheck ‘Use same settings as parent site’ before trying to change the ONVIF Profile Token.

Note: if you’ve configured the Vemotion Streaming Server to allow single different stream for multiple users then you only need to change the login details in “Device Access”.

- You now have two Vemotion Streams added.

Method B – Web Service Discovery on multiple ports

- Start multiple ports on the VProxy

Edit the vproxy.cfg file adding a list of ports, each for the same server e.g.

-w 80 vma 81 vma 82 vma 83 vma 84 vma

Don’t forget to restart the VProxy

- Restart indigoVision Control Centre.

This is required so that any discovered devices are cleared from the cache (which sometimes prevents new devices being discovered).

- IndigoVision should detect all configurations as individual Cameras

- You can now add Vemotion devices individually or as a block from “Top Site” – “Add Devices…”.

No Need to enable cloning.

Follow steps 5 & 6 in Method A to set username/password and desired stream.

Method C – Multiple IP Addresses (Linux only)

- You will need to configure multiple IP addresses

Check which connection you are using and then add more IP address with nmcli

~# nmcli c NAME UUID TYPE DEVICE MyConnection d79009f2-47f2-3012-8a54-4cdddf38b32a ethernet eth0 WiredDHCP 9c9191ea-bfec-4346-aba9-3d6eb0a70644 ethernet -- pgtestwifi ead30d88-aa2e-4c79-bdef-69869b410a40 wifi -- ~# nmcli c mod “MyConnection” +ipv4.addresses 192.168.1.10/24,192.168.1.11/24,192.168.1.12/24 ~#

- Manually add cameras using each of the additional static IP addresses that you added.

Use steps 3 to 7 from main Method A.