# RTSP Proxy

## Version 0.9.9.0

Vemotion Installation Instructions & Admin Guide.

Table of Contents

RTSP Proxy	1

Version 0.9.9.0	1

Installation	3

Files:	3

Location	3

Service	3

Running RTSPProxy	4

Config File	4

Command Options	4

Assisting NAT Traversal (Interleaving)	4

Security	4

Connecting to the RTSP Proxy	5

Connecting a Browser	5

Connecting an RTSP Client	5

Connecting a Telnet Client for Admin	5

Changing Logging	6

Stopping RTSP Proxy	6

Closing a Telnet Session	6

Sending PTZ / Camera / Profile Change Commands	6

PTZ Controls	6

PTZ Commands	7

Camera Controls	7

Encoding Profile Controls	7

Editing Your Own HTML Files	8

$tw{varname}	8

$tw{varname?true:false}	8

RTSP Proxy Variables	9

Glossary	10

# Installation

## Files:

RTSPProxy.exe	-	The executable.

index.html		-	Main camera control page for Web-browser based camera control.

ptz.html		-	Dummy page returned when requesting PTZ controls.

profile.html		-	Dummy page returned when requesting encoding-profile changes.

camera.html		-	Dummy page returned when requesting different cameras.

## Location

Create a directory under the standard Program Files destination called RTSPProxy. Copy RTSPProxy.exe here. Create a sub-directory called “web” and copy all .html files supplied with the RTSP Proxy to the web directory.

## Service

To install the RTSP Proxy as a service:

- Open a command prompt

- “cd” into the directory RTSPProxy is installed.

- Type “RTSPProxy install”

To start the service type “RTSPProxy start”. Note: You may need to run RTSPProxy normally from the command line to configure how it starts. See “Running RTSPProxy” for details on how to do this.

To stop the service type “RTSPProxy stop”

To remove the service type “RTSPProxy remove”

Use Windows Service Manager to further control how the service runs.

# Running RTSPProxy

## Config File

When every you run RTSPProxy.exe from the command line, it creates a file called RTSPProxy.cfg. This file contains all the command line options the you supplied so that next time it is executed as a service it knows what options to apply.

## Command Options

You can see the command option by typing “RTSPProxy /?” or “RTSPProxy -h”.

This is the output you should see:

You can run rtspproxy as a service with a single 1st parameter:

rtspproxy <install | remove | start | stop>

Vemotion RTSP Proxy version 0.0.5.7 options:

-a <ip.addr> Our IP Address for RTSP connections. (default 127.0.0.1)

-t <port>    Our TCP port for RTSP connections. (default 554)
  -w <port>    Our Web http port for camera control. (default 80)

-u <port>    UDP port for RTP Responses. (default 5544)

-l <licence> Vemotion licence string.

-i           Force Interleaved RTP over RTSP solves some NAT issues

-s <suffix>  Suffix for all Vemotion Server addresses e.g. .vemotion.com:4444

-d           Allow dots (.) in Vemotion Server addresses !Security risk!

-e <seconds> RTSP Expriry period.

If parameters are left completely blank the last used parameters

will be loaded from the config file.

For the RTSP Proxy to allow connections from anywhere other than from localhost you must supply the “-a <ip.address>” parameter for the IP address that the RTSP Proxy listens on. You will also need the “-l <licence>” parameter for the Vemotion licence string. See your Vemotion Server configuration for this.

### Assisting NAT Traversal (Interleaving)

Opening one-way UDP connections (for RTP video/audio) from behind a NAT router can be troublesome. By specifying the “-i” option, the RTSP Proxy will always reject requests for RTP over a UDP connection with a “461 Transport Unsupported”. This will result in the RTSP client requesting the stream again  with the RTP being transmitted over the existing RTSP connection.

Even if the RTSP Proxy is not configured with Interleaving forced on, it will always honour requests for RTP interleaved over the RTSP connection.

### Security

The Vemotion server that the RTSP Proxy connects to, is specified in the URL of each connection. To prevent external web connections specifying that the Proxy connect to any address of their choosing, the RTSP Proxy will only accept the Vemotion server name specified up-to the first dot '.'. So specifying “attack.this.google.com” will only result in the Proxy trying to resolve a server called “attack” which would not exist. It is possible to disable this security feature !AT YOUR OWN RISK! With the “-d” option.

With the “-s <suffix>” used as “-s .vemotion.com” the request to connect to server “attack.this.google.com” would result in “attack.vemotion.com” being resolved by the RTSP Proxy. This safely ties all Vemotion server connections to the vemotion.com domain.

# Connecting to the RTSP Proxy

The URL for connecting an RTSP client or Web-Browser is very similar:

[rtsp|http]://[user[:password]@]proxy.addr[:port]/vemotion.server/feed/[index.html]

To connect an RTSP or HTTP connection through to the Vemotion server you must have a Vemotion Server UserId and Password. See you Vemotion Server configuration for these.

Lets assume the following configuration:

RTSP Proxy – “proxy.lan”

Vemotion Server – “vemo.lan”

Video Feed – “internal”

User Name – “bob”

Password – “secret”

We'll assume that the RTSP Proxy is listening on port 80 (for http) and port 544 (for RTSP) in the examples below.

## Connecting a Browser

Given the above information, in your Web-Browser you would type:

http://proxy.lan/vemo/internal/index.html

You browser would then prompt you for a User and Password. Alternatively you can specify the User and Password in the URL with:

http://bob:secret@proxy.lan/vemo/internal/index.html

## Connecting an RTSP Client

Given the above information, in your RTSP-Client you would type:

rtsp://bob:secret@proxy.lan/vemo/internal/

## Connecting a Telnet Client for Admin

The RTSP Proxy is administered through a local telnet interface.

Given the above information and assuming the RTSP Proxy is listening on port 554; from the same machine that the RTSP Proxy is running on type:

telnet proxy.lan 554 (You could also use port 80).

Hit return until you see “---LOGGING SUSPENDED---” Logging is only suspended to your telnet session; logging continues as normal to the log files.

Hit return again and you will see “---LOGGING RESUMED---”. All logging entries will now be displayed on you telnet session.

For a list of available commands type “?” and hit return.

### Changing Logging

To View which logging classes are switched on type “class” or just “c” and hit return.

Relevant classes to this application are:
ERR – errors (default ON)
INFO – general information (ON)
MSG – shortened messages e.g. First line of RTSP/HTTP requests (ON)
FMSG – full messages e.g. Entire RTSP/HTTP messages & full Vemotion hex output (OFF)

To switch on a log-class e.g. full message logging type “c +fmsg”.

You can switch on a selection of classes for example with “c err +info +msg”

### Stopping RTSP Proxy

From the telnet interface you can stop the RTSP Proxy by typing “shutdown now”. Your connection will immediately terminate. You will need to use Windows Serivce Manager or “RTSPProxy start” to restart the RTSP Proxy.

### Closing a Telnet Session

You can close the telnet session either by closing the telnet window or by typing “quit”.

# Sending PTZ / Camera / Profile Change Commands

PTZ / Camera / Profile Change commands are all controlled through the same URL as the feed being viewed; with the exception that the viewing URL begins “rtsp” and the camera control URLs begin with “http”, and ends with a special control file and paramters.

You can send these controls to either port and even over the same existing RTSP connection, but they must be HTTP GET requests. (The standard request a browser makes when requesting a page.)

In the following examples we will assume the feed be viewed is:

rtsp://bob:secret@proxy.lan/vemo/internal/

## PTZ Controls

The URL for PTZ controls for the example feed would be:

http://bob:secret@proxy.lan/vemo/internal/ptz.html?<x>.<y>.<z>.<cmt>

Where:

x – is a numerical PTZ command (These are listed later)
y & z – are extensions to that command (usually speed controls)
cmt – is an optional comment (useful for debugging traces)

On receipt of a PTZ move/zoom/iris requst (1-15). The RTSP Proxy returns the file ptz.html with a “200 OK”. It will also set the HTTP connection: state to “keep-alive”. The connection may be closed at this time if necessary.

On receipt of a PTZ Stop (18) command RTSP Proxy will return the file ptz.html with a “200 OK”. It will set the http connection: state to “close”. The connection may be kept open at this time if necessary.

It is not permissible to send requests to different Vemotion servers or feeds over the same RTSP connection.

### PTZ Commands

| Command | Argument 1 | Argument 2 |
| --- | --- | --- |
| 1 (Up) | 0 – 100 (speed %) |  |
| 2 (Down) | 0 – 100 (speed %) |  |
| 3 (Left) | 0 – 100 (speed %) |  |
| 4 (Right) | 0 – 100 (speed %) |  |
| 5 (UpLeft) | 0 – 100 (pan speed %) | 0 – 100 (tilt speed %) |
| 6 (UpRight) | 0 – 100 (pan speed %) | 0 – 100 (tilt speed %) |
| 7 (DownLeft) | 0 – 100 (pan speed %) | 0 – 100 (tilt speed %) |
| 8 (DownRight) | 0 – 100 (pan speed %) | 0 – 100 (tilt speed %) |
| 9 (ZoomIn) | 0 – 100 (speed %) |  |
| 10 (ZoomOut) | 0 – 100 (speed %) |  |
| 11 (FocusNear) |  |  |
| 12 (FocusFar) |  |  |
| 13 (IrisOpen) |  |  |
| 14 (IrisClose) |  |  |
| 15 (Home) |  |  |
| 16 (GotoPreset) | preset number (1based) |  |
| 17 (SetPreset) | preset number(1based) |  |
| 18 (Stop) |  |  |

## Camera Controls

The URL for Camera selection for the example feed would be:

http://bob:secret@proxy.lan/vemo/internal/camera.html?<x>.<cmt>

Where:

x – is a numerical Camera ID
cmt – is a comment (useful for debugging traces)

You can find the Camera Ids by creating your own .html/.txt/.xml file and using the $tw{CAMERA_ID[0-9]} and $tw{CAMERA_NAME[0-9]} directives. See “Editing Your Own HTML Files” below for details.

## Encoding Profile Controls

The URL for Encoding Profile selection for the example feed would be:

http://bob:secret@proxy.lan/vemo/internal/profile.html?<x>.<cmt>

Where:

x – is a numerical Profile number
cmt – is a comment (useful for debugging traces)

You can find the profile descriptions by creating your own .html/.txt/.xml file and using the $tw{PROFILE_NAME[0-9]} directive. See “Editing Your Own HTML Files” below for details.

# Editing Your Own HTML Files

You may wish to edit the default index.html file provide. The RTSP Proxy will send out any file in the html directory. To experiment copy index.html to yourfile.html and point your browser (using the previous examples configuraiton) at http://proxy.lan/vemo/internal/yourfile.html

In doing this you can have multiple profiles for how your RTSP feeds are viewed and control. One file may be a full browser based control (like the supplied index.html), another may be an XML description of what features the camera supports for an RTSP client to process.

## $tw{varname}

When sending an html file the RTSP Proxy parses the file for specific identifiers which can be replaced depending on the logic required.

Wherever the RTSP Proxy finds “$tw{varname}” it checks varname against internal and URL based variables. URL variables being anything after the “?” e.g. 
“...index.html?myvar=1?anothervar=fish”

In this instance wherever the file index.html contains “$tw{anothervar}” it would be replaced with “fish”.

## $tw{varname?true:false}

It is also possible to apply some boolean logic to the substitution with the following syntax:

$tw{varname?truetext:falsetext}

Using this syntax if varname numerically evaluates to anything non-zero then the truetext is used otherwise the falsetext is used.

So given the following URL:“...index.html?hidevid=1”

If index.html contained the following text:

<div style="display:$tw{hidevid?none:inline}">

It would be transmitted as this:

<div style="display:none">

## RTSP Proxy Variables

| Substitution Name | Returned data |
| --- | --- |
| URLs |  |
| RTSP_URL | "rtsp://user:pass@Proxy:port/VemotionServer:VemotionPort/Feed/" |
| HTTP_URL_SERVER | "http://Proxy:port/VemotionServer:VemotionPort" |
| PTZ booleans |  |
| PTZ_UP | 0 / 1 if The Camera supports that PTZ action |
| PTZ_DOWN | 0 / 1 if The Camera supports that PTZ action |
| PTZ_LEFT | ... |
| PTZ_RIGHT |  |
| PTZ_STOP |  |
| PTZ_UPRIGHT |  |
| PTZ_UPLEFT |  |
| PTZ_DOWNRIGHT |  |
| PTZ_DOWN_LEFT |  |
| PTZ_SPEED |  |
| PTZ_PRESETGO |  |
| PTZ_PRESETSET |  |
| PTZ_ZOOMIN |  |
| PTZ_ZOOMOUT |  |
| PTZ_FOCUSNEAR |  |
| PTZ_FOCUSFAR |  |
| PTZ_IRISCLOSE |  |
| PTZ_IRISOPEN |  |
| Camera Info |  |
| CAMERA_NAME[0-9] | The camera text name. |
| CAMERA_EXIST[0-9] | 0 / 1 depending if the camera exists |
| CAMERA_ID[0-9] | A number between 0 and 255 |
| CAMERA_SELECTED[0-9] | 0 / 1 depending if the camera is currently selected |
| Profile Info |  |
| PROFILE_NAME[0-9] | The encoding profile description text. |
| PROFILE_EXIST[0-9] | 0 / 1 depending if the profile exists |
| PROFILE_SELECTED[0-0] | 0 / 1 depending if the profile is currently selected |
| Feed Info |  |
| FEED_NAME[0-9] | Text name of feed to use in a URL. |
| FEED_EXIST[0-9] | 0 / 1 depending if the feed exists |
| FEED_SELECTED[0-9] | 0 / 1 depending if the profile is currently selected |
| General |  |
| VERSION | The Version number of the RTSP Proxy |
| CONNECTIONS | The number of current active RTSP / HTML connections |
| PARAMETERS | The full URL parameters string |

# Glossary

HTTP – Hypertext Transfer Protocol. How web pages are delivere.

RTP – Real-Time Transport Protocol.  A streaming format for video and audio.

RTSP – Real Time Streaming Protocol. A means of negotiating streaming of RTP.

PTZ – Pan Tilt Zoom.

TCP – Transmission Control Protocol. A lower level stream connection used to transport RTSP and HTTP.

UDP – User Datagram Protocol. A simple protocol often used to transport RTP.

URL – Uniform Resource Locator. How we describe where to find an RTSP or HTTP feed.